Q2.1) For the Following C Statement, what is the corresponding MIPS assembly code? Assume that the variables f, g, h, and I are given and could be considered 32 bit integers as declared in a C program. Use a number of MIPS assembly instructions.

f = g + ( h – 5);

ANS: -  Lets us assume the variables f, g and h are assigned to the registers $t1, $t2 and $t3 respectively.

Addi $t0, $t3, -5 	#temporary register $t0 contains h-5

Add $t1, $t2, $t0 	#f gets g + $t0 which is g + (h – 5);

Q2.2) For the following MIPS assembly instructions below, what is the corresponding C Statement.

Add f, g, h

Add f, i, f

ANS) 	f = g + h,

f = i + f

Q2.7) Show how the value 0xabcdef12 would be arranged in memory of a little-endian machine and a big-endian machine. Assume the data is stored starting at the address 0.

Ans:-

In little endian, we store the least significant byte in the smallest address.

| Address | Value |
| --- | --- |
| 0 | 12 |
| 1 | ef |
| 2 | cd |
| 3 | ab |

In big endian, we store the most significant byte in the smallest address.

| Address | Value |
| --- | --- |
| 0 | ab |
| 1 | cd |
| 2 | ef |
| 3 | 12 |

Q2.8) Translate 0xabcdef12 into decimal.

ANS: -  = (2 * 160) + (1 * 161) + (15 * 162) + (14* 163) + (13 * 164) + (12 * 165) + (11 * 166) + (10 * 167)

= 2 + 16 + 3840 + 57344 + 851968 + 12582912 + 184549376 + 2684354560

= 2882400018

Q2.9) Translate the following C code to MIPS. Assume that the variables f, g, h, I and j are assigned to the registers $s0, $s1, $s2, $s3 and $s4 respectively. Assume that the base address of the arrays A and B are in the registers $s6 and $s7 respectively. Assume that the elements of the arrays A and B ae 4 byte words,

B[8] = a[i] + a[j];

ANS: -

sll  $t1, $s3, 2	#Temp reg $t1 = i * 4

To get the address of A[i] we need to add $t1 to the base of the A in $s6

Add $t1, $t1 , $s6	# $t1 = address of the A[i]

Now we can use that address to load A[i] into a temporary register

lw $t0, 0($t1)	# temp Reg $t0 = A[i]

sll  $t3, $s4, 2	#Temp reg $t3 = j * 4

To get the address of A[j] we need to add $t3 to the base of the A in $s6

Add $t3, $t3 , $s6	# $t3 = address of the A[j]

Now we can use that address to load A[j] into a temporary register

lw $t2, 0($t3)	# temp Reg $t2 = A[j]

Now add the contents of A[i] and A[j]

Add $t4, $t0, $t2	#$t4 = $t0 + $t2

Store the contents into B[8]

sw $t4, 32($s7)	#stores $t4 into B[8]